**************************************************************************

**                                                                      **

**                GARDOWN-GPS-MENU Version 2.0 MANUAL                   **

**                                                                      **

**************************************************************************

1. Introduction

---------------

GARDOWN-GPS-MENU is an extension for the GARDOWN software. It was written

to have a  convenient interface  for GARDOWN  plus the  ability to easily

modify data of the  GARMIN GPS unit.  Details how to  modify  data can be

found in the second section of this documentation (GARDOWN MANUAL).

2. Hardware and Software Requirements

-------------------------------------

GARDOWN and the  GARDOWN-GPS-MENU  will work with any PC-XT or PC-AT and

MS-DOS 3.3 or later. Both applications need very little system resources

and no specific configuration of the PC. They will also work with EGA/CGA

screens and LC-Displays.

That means both applications will also work on palmtop computers like the

HEWLETT  PACKARD 200LX and on  palmtops like the PSION 5 Series or the HP

JORNADA 700 Series in combination with a MS-DOS Emulator (XTM on the Psion

or Pocket-DOS on the HP Jornada 710/720/728).

GARDOWN and GARDOWN-GPS-MENU will only work with GARMIN GPS units and has

been tested with following models:  GPS 12,  GPS 12XL,  GPS 12CX,  GPS II,

GPS IIPlus, GPS III, GPSIIIPlus, GPS V. Older models that work are GPS 38,

GPS 45, GPS 45XL GPS 65 and GPS 75. eMap and eTrex series will definitely

not work with GARDOWN due to different interface standards.

3. Connection to a PC or a Palmtop-PC

-------------------------------------

Cable connection is a very important issue. Many problems result in faulty

connections. Connecting a GARMIN to a STANDARD-PC is quite simple: Plug the

GARMIN cord into COM1 and connection is possible.

If a Palmtop-PC is used in most cases a combination of GARMIN cable and the

serial cable of the Palmtop-PC is used. On the HP200LX, the  HP JORNADA 700

Series, the PSION 5 Series and many other Palmtop-PCs the serial link cable

does not have the standard pin layout. To  overcome this  problem a special

cable is required or a so called "Null Modem Adaptor" has to be added. Null

Modem Adaptors are available at the computer store.

Layout of a 9-pin SUB-D-Nullmodem-Adaptor:

1 --connect to-- 1

2 --connect to-- 3

3 --connect to-- 2

4 --connect to-- 6

5 --connect to-- 5

6 --connect to-- 4

7 --connect to-- 8

8 --connect to-- 7

9   leave open   9

Connection looks like this:

------   ------------   -----------------   ------------   ----------

GARMIN-> GARMIN CABLE-> NULLMODEM-ADAPTOR-> SERIAL-CABLE-> PALMTOP-PC

------   ------------   -----------------   ------------   ----------

4. Functions of GARDOWN-GPS-MENU

--------------------------------

MAIN MENU ----- DOWNLOAD MENU   --- DOWNLOAD WAYPOINTS FROM GPS

--- DOWNLOAD TRACKS FROM GPS

--- DOWNLOAD ROUTES FROM GPS

--- DOWNLOAD PROXIMITY WAYPOINTS

----- UPLOAD MENU     --- UPLOAD WAYPOINTS TO GPS

--- UPLOAD TRACKS TO GPS

--- UPLOAD ROUTES TO GPS

--- UPLOAD PROXIMITY WAYPOINTS

----- EDIT GPS-DATA   --- EDIT WAYPOINTS

--- EDIT TRACKS

--- EDIT ROUTES

--- EDIT PROXIMITY WAYPOINTS

----- GPS INFORMATION --- SHOW GARMIN DATE AND TIME

--- SHOW GARMIN MODEL CODE

--- TURN OFF GARMIN

----- EXIT

DOWNLOAD MENU  is to get data from the GPS into the PC. Data will be saved

-------------  immediately after transfer in GARDOWN-subdirectory. No data

in the GPS will be changed or moved.

UPLOAD MENU    is to get data  from the PC into the  GARMIN GPS. It is

-----------    recommended to erase all data in the GARMIN manually to

ensure enough space for new data and to avoid double entries.

EDIT GPS-DATA  Data stored in  GARDOWN-subdirectory can be edited. This is

-------------  to add or remove entries in the files for Waypoints, Tracks,

Routes and Proximity Waypoints. A description of file syntax is included in

the GARDOWN Manual below.

GPS-INFORMATION  displays time of GARMIN unit, the model code and has an

---------------  option to switch off the GARMIN.

EXIT           terminates GARDOWN-GPS-MENU.

----

5. Predefined Settings

----------------------

GARDOWN-GPS-MENU was mainly made for use with a HP200LX palmtop-PC. This

is the reason that many settings are predefined and fixed.

Serial communication:  COM1 (the first serial port, HP200LX has only COM1)

Waypoints file:        WAYPOINT.GAR (text file in GARDOWN subdirectory)

Tracks file:           TRACKS.GAR   (text file in GARDOWN subdirectory)

Routes file:           ROUTES.GAR   (text file in GARDOWN subdirectory)

Proximity Waypoints:   PROXWP.GAR   (text file in GARDOWN subdirectory)

***************************************************************************

**                                                                       **

**                           GARDOWN MANUAL                              **

**                                                                       **

***************************************************************************

***************************************************************************

* This part is the original GARDOWN Documentation made by M.J. Montgomery.*

*          Copyright (c) 1995 - 1998 by Mike J. Montgomery                *

***************************************************************************

GARDOWN will Download (to the PC) and Upload (to the GARMIN(sm) GPS)

Waypoints, Tracks, Routes, and Proximity Waypoints held in the GARMIN

GPS using Serial Port COM1 or COM2.

GARMIN (sm) is a trademark of GARMIN International

GARDOWN has been tested with GARMIN 45, 65, 75, 45XL, GPS-II.

IMPORTANT

---------

The Datum set in the GPS Navigation page should match that of the map(s)

that you are using or you could get very lost.

Your GPS is only accurate to 15 meters (49 feet) even with Selective

Availability  turned off.  With S.A. on it is 100 meters (330 feet).

Your GPS does not replace a good map and knowing how to use it.

If you are  uploading Tracks or  Waypoints, you will not be able to

follow the track exactly, don't try to.  Waypoints will also be off

by some amount.

You should (but it is not always necessary to) delete all Waypoints

and the Track in your GPS when uploading.  You don't have to delete

the  waypoints if you have  room for the  number in the upload file.

You do not have to clear the track if you do not mind the potential

loss of early track data. The GPS will only hold a limited amount of

track data so you also have to be careful of accumulating a track that

overlays the track in the upload file.

USAGE

-----

GARDOWN {opn} *infile *outfile**

{   } indicates that one or more is required

*

* indicates that the enclosed item is optional

When  GARDOWN starts, it will tell you the  interface mode that you

should have your GARMIN GPS set to. See the following chart for the

required interface modes for the various GARDOWN operations.

Pressing any key will return you to DOS if:

- Nothing is happening

- you might have the wrong interface mode set, make sure it is

set as below

- your position is not changing (i.e., you are in SIMULATOR mode)

- your GARMIN GPS is not connected

- your GARMIN GPS is turned off

- you want to terminate an operation before it completes

- you might want to do this if you are transferring lots of waypoints,

routes, and/or tracks and you realize that they are not what you

really want

Both the Datum and GPS version are automatically send back when doing a

download with GARDOWN. The -D operation can be used to have GARDOWN

perform a conversion to a different datum.

Operations are 	proceeded by a - (dash) and may be entered in either

UPPER or lower case.  Parameters that may be associated with an operation

immediately follow the operation (i.e. -s2 for serial port 2).

Multiple GRMN/GRMN interface mode operations may be specified on a single

GARDOWN invocation except that a single operation should not specified more

than one time.

Opn  Par Description                        Port  Datum  File(s)   Ifce Mode

---  --- ---------------------------------- ----  -----  --------  ---------

-A       Output Date format for GPSS        n/a  n/a    *Output*  NMEA/NMEA

-B       Download from a Micrologic GPS.    Yes   n/a   *Output*

-C       Download GARMIN time               Yes   n/a   *Output*  GRMN/GRMN

-D    ?  Show Datum Numbers                 n/a   n/a     n/a        n/a

-D   nnn Set Coordinate Datum               n/a   n/a     n/a        n/a

-F   nnn Set the format to LL UTM OSGB GRID n/a   n/a     n/a        n/a

-G       Convert ODB file to GARDOWN (D75)  n/a   n/a     Both       n/a

-H       Show GARDOWN help                  n/a   n/a     n/a        n/a

-J   nnn Convert GARDOWN file to ODB (D75)  n/a   Yes     Both       n/a

-M       Upload Extend WPTS to NMEA device  Yes   Yes    Input    NMEA/NMEA

-N       Log NMEA 183                       Yes   n/a     n/a     NMEA/NMEA

-O       Turn off GARMIN                    Yes   n/a     n/a     GRMN/GRMN

-P       Download Proximity Waypoints       Yes   Yes   *Output*  GRMN/GRMN

-Q       Download Current Position          Yes   Yes   *Output*  GRMN/GRMN

-R       Download Routes                    Yes   Yes   *Output*  GRMN/GRMN

-S    n  Set Serial Port Number             n/a   n/a     n/a     As Needed

-T       Download Track                     Yes   Yes   *Output*  GRMN/GRMN

-U       Upload file (Datum set in file)    Yes   Yes    Input    GRMN/GRMN

-V       Get code level                     Yes   Yes   *Output*  GRMN/GRMN

-W       Download Waypoints                 Yes   Yes   *Output*  GRMN/GRMN

-X   nnn Change file datum                  n/a   Yes     Both       n/a

-Y       Upload Waypoints to NMEA device    Yes   Yes    Input    NMEA/NMEA

OPERATIONS

----------

-A      Output Date format for GPSS

Date format is presented in a format compatible with the

GPSS program. See below for the format.

-C      Download GARMIN time

This is the time without adjustment for the OFFSET.

-B   	Download Extended Waypoints, Routes & track from a Micrologic GPS.

-D?     Show Datum Numbers

Displays a list of Datum numbers and definitions.

-Dnnn   Set Coordinate Datum

Sets the  Datum number to use for the subsequent option.

Defaults to 100 (WGS 84).  Valid are 0-103.  The GARMIN

only  handles one datum at a time.  It should be set to

the same as the maps that you are using.

-Fnnn   0 being the normal DD MM.MMMM format

1  Ordnance Survey Great Britain (ODB)

2  Universal Transverse Mercator (UTM)

See below for the format of these positional representations.

-G      Convert ODB file to GARDOWN (D75)

Convert an ODB file used with Psion s3a program to call tidestream

into a GARDOWN file.  Datum is set to 75 (Ordnance Survey Great

Britain).

-H      Show GARDOWN help

Same as invoking GARDOWN with no parameters

-Jnnn   Convert GARDOWN file to ODB (D75)

Convert a GARDOWN file into an ODB file used with Psion s3a

program to call tidestream.  Datum is set to 75 (Ordnance

Survey  Great Britain) by default.  Produces one file for

each route and one .ODB file.

-M      Upload Extended Waypoints,Routes & track to a Micrologic GPS.

*******************   W A R N I N G    ****************************

*  This IS NOT intended to be used to upload waypoints to a       *

*  Garmin GPS.  On the GARMIN GPS-II it causes a lockup requiring *

*  removal of the batteries.  After reinserting the batteries,    *

*  ALL stored data is lost and the GPS is set to the state as     *

*  received from the factory!!                                    *

*      Position = N39 00.000, W095 00.000, ALT 1081'              *

*      Re-initialization is required                              *

*******************************************************************

Send the waypoints in the input file to a NMEA device using the

$PMCLF NMEA sentence.  This sentence consists of:

$PMCLF,lat,latq,long,lonq,wptnam,?unk1,?time,?date,?unk2,comment

where: lat     is the latitude in the form DDMM.MMM

latq    is N or S

long    is the longitude in the form DDDMM.MMM

longq   is E or W

wptnam  is the name of the waypoint

unk1    is unknown

time    is (maybe) the time that the waypoint was created

(format= UTC, hh:mm)

date    is (maybe) the date that the waypoint was created

(format= DDMMYY)

unk2    is unknown

comment is the comment from the waypoint, truncated to 9

characters

-N      Log NMEA 183

Logs NMEA 183 data

This is currently being worked on.  You can log the output by

redirecting to a file (GARDOWNB -N > nmea.fil).

-O      Turn off GARMIN

This happens really fast, you can't push a key to stop it.  It does

NOT go  through the normal  GARMIN shutdown count. By a nice way to

turn off the garmin in a hurry.

-P      Download Proximity Waypoints

Get Proximity Waypoints from the GARMIN GPS.

If proximity waypoints are not supported on your GARMIN GPS, the

program will wait, you have to push a key to end the program.

-Q      Download Current Position

Download your current position from the GARMIN GPS.  If you are not

moving, nothing is transferred to the PC.  This will be display in

lat long only. This must be terminated by pushing a key to end the

program.

-R      Download Routes

Download all routes from the GARMIN GPS.

-Sn     Set Serial Port Number

Sets the Serial Port Number to use for the subsequent option.

Defaults to 1 (COM1).

-T      Download Track

Download the track from the GARMIN GPS.

-U      Upload file (set Datum in the file)

Upload the information in a  GARDOWN  file to the GARMIN GPS.

File(s) created by GARDOWN downloads are directly uploadable.

You should  set the datum to  what you used for the download.

If you are uploading new data, make sure that the datum is

the same as for your map.  See below for the format and

content of the text file to upload.

-V      Get code level

Downloads the GARMIN GPS code version number.  This contains

both the GARMIN  GPS type and version number of the internal

GARMIN GPS code.

-W      Download Waypoints

Download all waypoints from the GARMIN GPS.

-Xnnn   Change file datum

Change the Coordinate of the input file to the datum number

specified in the output file.

-Y      Upload Waypoints to NMEA device

Send the waypoints in the input file to a NMEA device using the

$YMWPL NMEA sentence.  This sentence consists of:

$YMWPL,lat,latq,long,lonq,wptnam

where: lat     is the latitude in the form DDMM.MMM

latq    is N or S

long    is the longitude in the form DDDMM.MMM

longq   is E or W

wptnam  is the name of the waypoint, truncated at 4 characters

Examples

--------

Download Track using COM2. WGS 84 datum is used by default:

gardowna -s2 -t

Download Waypoints using Datum 75, Ord Srvy Grt Britn & output

to file my.way. COM1 is used by default:

gardowna -w -d75 my.way

Download Routes using NAD 27 CONUS, COM1 to file mytrip.rte:

gardowna -s1 -d61 -r mytrip.rte

Download everything (clock, proximity waypoints, routes, track,

code level, waypoints)

gardowna -s2 -d75 -c -p -r -t -w

The sequence that these are presented in the download data is:

-D  Coordinate Datum - sent by default as WGS84

-V  GARMIN Type and Version - sent by default, not necessary

to specify

-P  Proximity Waypoints

-R  Route Data

-C  GARMIN Time

-T  Track Data

-W  Waypoint Data

At the end of this file, there is a sample DOS batch file that

will download all pertinant information contained in the GARMIN

GPS for a particular excursion.

Text File Format

----------------

The text file being used for upload into the GARMIN GPS is always less

than or equal to 80 characters per line.

Blanks and . (period/dot) and : (colon) are required and _ (underscore)

is a place-holder.

Positional (coordinate) data is usually expressed in degrees and decimal

minutes out to four digits.

HDD MM.MMMM

|-- -------

||     |

||     +----------- Decimal Minutes of Latitude

|+----------------- Degrees of Latitude

+------------------ Hemisphere (N or S)

HDDD MM.MMMM

|--- -------

| |     |

| |     +---------- Decimal Minutes of Longitude

| +---------------- Degrees of Longitude

+------------------ Hemisphere (E or W)

If -D102 is specified, the positional (coordinate) data

is expressed in the Ordnance Survey of Great Britain (ODB) format:

ZZ EEEEE NNNNN

-- ----- -----

|    |     |

|    |     +-------- Northing (5 digits)

|    +-------------- Easting (5 digits)

+------------------- ODB Zone (two letters)

If -D103 is specified, the positional (coordinate) data

is expressed in the Universal Transverse Mercator (UTM) format:

ZZ L EEEEEEE NNNNNNN

-- | ------- -------

|  |    |       |

|  |    |       +--- Northing in meters from the Equator

|  |    +----------- Easting in meters relative to the Zone

|  +---------------- Latitude Zone Letter

+------------------- Longitude Zone Number

As can be seen from the above, the position information can vary

in length. The position information in the following text file

formats will be shown as: _position_data_

Timestame (date and time) is usually expressed as:

DDD MMM dd hh:mm:ss YYYY

--- --- -- -- -- -- ----

|   |  |  |  |  |   |

|   |  |  |  |  |   +------- Year (nnnn)

|   |  |  |  |  +----------- Seconds (nn, 00-59)

|   |  |  |  +-------------- Minutes (nn, 00-59)

|   |  |  +----------------- Hour (nn, 00-24)

|   |  +-------------------- Day of Month (01-31)

|   +----------------------- Month (Jan, Feb, Mar, Apr, May, Jun,

|                            Jul, Aug, Sep, Oct, Nov, Dec, Cur).

|                            If 'Cur', then the Current PC time

|                            will be used. (This depends on the

|                            GARDOWN software level and may not

|                            work in all cases.  If it does not

|                            work, the date will be:

|                            Sun Dec 31 00:00:00 1989

+--------------------------- Day of Week (Sun, Mon, Tue, Wed, Thu,

Fri, Sat).  Not needed but must be padded.

If the -A parameter is included, then the timestamp is converted

to the format that the GPSS program can use:

DD-MMM-YY hh:mm:ss

-- --- -- -- -- --

|   |  |  |  |  |

|   |  |  |  |  +----- Seconds (nn, 00-59)

|   |  |  |  +-------- Minutes (nn, 00-59)

|   |  |  +----------- Hour (nn, 00-24)

|   |  +-------------- Year (Last two digits, GPSS must assume 19YY)

|   +----------------- Month (JAN, FEB, MAR, APR,MAY , JUN, JUL, AUG,

|                      SEP, OCT, NOV, DEC)

+--------------------- Day of Month (01-31)

As for positional information, timestamp information is also of variable

length.  Therefore, timestamp information in the following file formats will

be shown as:

_timestamp_data_

When you upload, BEFORE you upload any routes, waypoints, or tracks, you

should set the datum properly.

You do not have to separate the waypoints from the routes.  The waypoint (W)

or named waypoint (WR) definition can be contained within a route (R nn) ...

(E  nn) definition.  If (WR) is used, the WPNAME must be defined in the upload

file with a waypoint (W) line somewhere.

Any lines other than those defined below are treated as comments and ignored

in the upload file.

Datum Number:

D  nnn

|  ---

|   |

|   +-- datum number, left justified

+------ line must start with an UPPER case D and two spaces

Waypoints:

W  WPNAME _position_data_ _timestamp_data_ WAYPOINT_COMMENT____

|  ------ --------------- ---------------- --------------------

|    |           |               |                  |

|    |           |               |   Comment -------+

|    |           |               +-- Creation Timestamp on GPS

|    |           +------------------ Location

|    +------------------------------ Six character Identification

+----------------------------------- line must start with an

UPPER case W and two spaces

Routes:

Start a Route:

R  01 MY ROUTE 1

|  -- --------------------

|  |           |

|  |           +-- Comment

|  +-------------- route number must be two digits

+----------------- line must start with an UPPER case

R and two spaces

Defined Waypoint added to Route:

WR WPNAME

-- ------

|    |

|    +-- Six character waypoint name or number

+------- line must start with an UPPER case WR and one space

This will take waypoint WPNAME from the waypoint in

the same upload file

Defining a Waypoint in Route:

W  WPNAME _position_data_ _timestamp_data_ WAYPOINT COMMENT____

(same a waypoints above)

Ending a Route:

E  01

|  --

|  |

|  +-- Route Number (two digits)

+----- line must start with an UPPER case E and two spaces

Proximity waypoints:

P  WPNAME  nn.n

|  ------  ----

|    |      |

|    |      +-- two digits and one decimal place, in Nautical miles

|    +--------- Six character waypoint name or number

+-------------- line must start with an UPPER case P and two spaces

This will take waypoint WPNAME from the waypoint in

the same upload file

Tracks:

T  _position_data_ _timestamp_data_

|  --------------- ----------------

|         |               |

|         |               +-- Creation Date

|         |                   (e.g. Sat Feb 04 10:29:23 1995)

|         +------------------ Location

+---------------------------- line must start with an UPPER

case T and two spaces

Current Position:

_position_data_ *_timestamp_data_

--------------- |----------------

|        |       |

|        |       +-- Position timestamp

|        |           (e.g. Sat Feb 04 10:29:23 1995)

|        +---------- Tab (X'09') character

+------------------- Location

Position data can not be uploaded to the GARMIN but may

be downloaded from the GARMIN. The format on download is as shown.

SAMPLE BATCH FILES

------------------

Note:

You have to change all occurrences of GARDOWNx to the version of GARDOWN

that you are using.

REM ---------------------------- Cut here -------------------- SAMPLE01.BAT

@ECHO OFF

GOTO START

:INSTR

CLS

REM This batch file should be saved as GETALL.BAT

ECHO This batch file, *%0* invokes the GARDOWNx program to work with a GARMIN

ECHO GPS attached on the specified Serial Port, using the GRMN/GRMN protocol.

ECHO

ECHO It sets the Datum and gets Waypoints, Routes, and Tracks into separate

ECHO work files using the name specified with file Extensions:

ECHO       .STM for Start time, deleted at end of job

ECHO       .WPT for Waypoints, deleted at end of job

ECHO       .RTE for Routes, deleted at end of job

ECHO       .TRK for Tracks, deleted at end of job

ECHO       .ETM for End time, deleted at end of job

ECHO       .ALL for of the above, with the GARMIN GPS time, kept

ECHO

ECHO            +-------------- 1 : 1 = Port 1, 2 = Port 2

ECHO            I    +--------- 2 : 61=NAD27, 75=Ord GB, 99=WGS72, 100=WGS84

ECHO            I    I      +-- 3 : 5 characters max., to screen if omitted

ECHO            I    I      I

ECHO Format: %0 port datum *name*

ECHO

ECHO Invoked as: %0 %1 %2 %3 %4

ECHO

GOTO END

:START

IF p == p%1 GOTO INSTR

IF 1 == %1 GOTO GOTPORT

IF 2 == %1 GOTO GOTPORT

GOTO INSTR

:GOTPORT

IF d == d%2 GOTO INSTR

IF  61 == %2 GOTO GOTDAT

IF  75 == %2 GOTO GOTDAT

IF  99 == %2 GOTO GOTDAT

IF 100 == %2 GOTO GOTDAT

GOTO INSTR

:GOTDAT

CLS

ECHO Push any key to terminate GARDOWN if no response is received

IF f == f%3 GOTO SCREEN

REM      Port Datum Opn

ECHO                                                     Getting Start Time

ECHO

GARDOWNx -S%1 -D%2  -C  %3%2.STM

ECHO Start Time Above------------------------- Start Time Above >> %3%2.STM

CLS

ECHO                                                      Getting Waypoints

ECHO

GARDOWNx -S%1 -D%2  -W  %3%2.WPT

ECHO Waypoints Above -------------------------- Waypoints Above >> %3%2.WPT

CLS

ECHO                                                         Getting Routes

ECHO

GARDOWNx -S%1 -D%2  -R  %3%2.RTE

ECHO Routes Above -------------------------------- Routes Above >> %3%2.RTE

CLS

ECHO                                                          Getting Track

ECHO

GARDOWNx -S%1 -D%2  -T  %3%2.TRK

ECHO Track Above ---------------------------------- Track Above >> %3%2.TRK

CLS

ECHO                                                       Getting End Time

ECHO

GARDOWNx -S%1 -D%2  -C  %3%2.ETM

ECHO End Time Above ---------------------------- End Time Above >> %3%2.ETM

COPY %3%2.STM + %3%2.WPT + %3%2.RTE + %3%2.TRK + %3%2.ETM %3%2.ALL > NUL

DEL %3%2.STM > NUL

DEL %3%2.WPT > NUL

DEL %3%2.RTE > NUL

DEL %3%2.TRK > NUL

DEL %3%2.ETM > NUL

GOTO END

:SCREEN

REM      Port Datum Opn

GARDOWNx -S%1 -D%2  -C

GARDOWNx -S%1 -D%2  -W

GARDOWNx -S%1 -D%2  -R

GARDOWNx -S%1 -D%2  -T

GARDOWNx -S%1 -D%2  -C

:END

ECHO

REM ------------------------------ Cut here ------------------ SAMPLE01.BAT

Change History

--------------

Ver 0.02beta    30/3/95

-----------------------

Now will download Track, Waypoints & Routes

Now works without FPU

Packet bug fixed

Ver 0.03beta     3/4/95

-----------------------

Now using COM1 or COM2

Ver 0.04beta    18/4/95

-----------------------

Now you can upload waypoints to the GARMIN GPS from a text file

Ver 0.05beta    17/5/95

-----------------------

BUGFIX: comment field now converted to upper-case as the GARMIN GPS will

only accept upper-case

BUGFIX: Math calculation fix on some waypoints which were .9 out

BUGFIX: Better handshaking when uploading waypoints

BUGFIX: Well, I did one more but can't remember what

Ver 0.6Beta     22/5/95

-----------------------

You can now upload Routes from a text file

You can download & upload Proximity waypoints

You can download GPS time

You can download current position

If the month in a upload waypoint is Cur the date uploaded will be the PC

system time.

Ver 0.7Beta     30/7/95

-----------------------

You can now set datums at the command line type "GARDOWNx -D?", this will

list all datum numbers.

You can now upload tracks but there is a bug in my GARMIN 75.  To get around

this you must clear all tracks before uploading tracks.  You also

should reset track to loop after upload.

BUGFIX: Getting GPS time crash fixed.

Ver 0.08     14/Sept/96

-----------------------

GARDOWN now uses better command line options & better file handling.

You can now use GARDOWN to log NMEA 183 to a file or screen.

GARDOWN can upload waypoints to a Yemon ploter.

You can change a file of one datum to an other datum.

Ver 9.00      20/Feb/97

-----------------------

BUGFIX: Routes 10-30 now fixed

Ver 10.00     11/Mar/97

-----------------------

BUGFIX: Carrage return now put back.

BUGFIX: No longer leaves deg.cap file

Now can download waypoint to a NMEA 183 device but only in datum no 75

Ver 11.00b     21/Aug/97

------------------------

You can now download or upload with datums 102 British O/S Grid and

103 UTM grid.

Also so you can upload waypoints to an extended NMEA device i.e. Micrologic

ML-150 ML-250 GPS.  (Do NOT upload to a GARMIN GPS with this).

Changes are being made to the Log NMEA operation, these are incomplete at

this time.

Ver 12.00  05/Feb/98

--------------------

I have changed add a format command so OSGB & UTM can now have there own

Datum's. There for datum number 102 & 103 no longer work to get these format

-F0 Lat Long

-F1 OSGB

-F2 UTM

Ver 13d.00 19/Feb/98

--------------------

Bug fix for South lat Long to UTM

Programm Registration

---------------------

You are granted a no-cost license to use GARDOWN for any personal purpose

that does not involve the sale of any data produced with GARDOWN.  Within

this realm,  GARDOWN shall be considered to be freeware and may be freely

used and distributed at will.  If distributed, only the unaltered

GARDOWNx.ZIP file shall be transmitted to others.

All other uses shall be  considered to be  commercial in nature and subject

to a fifty ($50) licensing fee. The payment of the shareware license grants

the purchaser a single user license to use GARDOWN for  commercial purposes

that involve financial gain. Registered users will be notified by e-mail of

and supplied with  the latest  version free of  charge  ahead of any  other

posting.

Please send your registration to:

Mike Montgomery

6 East Hanningfield RD

Rettendon Common

Essex

England

CM3 8EG

E-Mail to:      mike@anali.demon.co.uk

WWW             anali.demon.co.uk

GARDOWN can be downloaded from:

http://www.anali.demon.co.uk/gardown.htm

**************************************************************************

**                                                                      **

**       COPYRIGHT (c) GARDOWN AND GARDOWN MANUAL 1995 - 1998 BY        **

**       MIKE J. MONTGOMERY,  COPYRIGHT (c) GARDOWN-GPS-MENU AND        **

**       GARDOWN-GPS-MENU-DOCUMENTATION 1997-2003 BY J. TROESZTER       **

**                                                                      **

**                                                                      **

**       All software used in combination with GARDOWN-GPS-MENU         **

**       is copyright protected.                                        **

**       Legal notice: Use this software at your own risk! The          **

**       producers of the software are not responsible for any          **

**       injury, damage, loss of data or anything else caused           **

**       by using one or all software-products.                         **

**       All software comes “as it is” without any warranty for         **

**       function or usability.                                         **

**************************************************************************