OS-INSTALLATION ON FUJITSU SIEMENS STYLISTIC 3400 / 3500 / LT-C500 / LT-C600

This is the complete How-To to install Windows 2000 or Windows XP on the Fujitsu Stylistic assuming a clean or new harddrive. It is also a work around not having a dock, or bootable floppy drive.

The basic principal is to prep your harddrive in a desktop PC, then boot the stylistic off of the new harddrive and do the Windows 2000/Windows XP setup on that drive, from that drive.

What you will need:

- A Fujitsu Stylistic 3400, LT-C500 or LT-C600 pen enabled slate computing device

- An IDE 40 pin to 44 pin 2.5” harddrive adapter

- Your Own Windows 2000/Windows XP CD-ROM and Cert. of Authenticity, specifically, the i386 directory and all its subdirectories.

- A USB Hub, Keyboard, and Mouse, or USB combo keyboard and mouse

- A desktop/tower PC that you have the ability to add another harddrive to

If the desktop is running Windows98, that’s all,

If the desktop is running Windows 2000 or Windows XP, it will need to have a working floppy drive that it can be booted from.

In general, these instructions assume you have some knowledge of administration/manipulation of your graphical, Windows 98, 2000, XP desktop.

Installation of basic operating system Windows 2000 or Windows XP

If you are using a Windows 2000/Windows XP desktop, a bootable Windows98 system disk will need to be made in preparation for the following operation:

Once you have extracted the Zip package, you will find several directories, one of which is “FloppyTools” in that directory run (double click) the program Floppy Image.exe. In the lower half of the screen, the section to Write Floppy, click the Browse button. You may have to browse back to the directory, and select the image setup.img.

Ensuring that a blank, or unnecessary floppy, disk is in the floppy drive, click start. When it’s finished, eject the disk and quit the program. We will get back to it later.

Now we will pick up the pace for a while

- Install new drive in desktop, preferably by itself on secondary IDE cable

- Partition / format FAT32 / Test, make the partition active!

- Reboot desktop with Windows 98 custom boot disk, if using Windows 98 just open a command prompt and do step 4 from within windows, skipping step five.

- SYS new drive

For example, in my PC the new drive shows up as c: because the Windows 98 boot disk cannot mount the NTFS volume that my Win2K resides on. Ensure that you are performing this operation on the new drive. You could really screw up your desktop in this step. In my situation the command looks like this:   A:>sys c:

- Reboot desktop to Windows 2000/XP

- Copy directories to new drive:  	i386		from your Win2K CD

Drivers		from the 3400setup.zip

Doco		from the 3400setup.zip

Also the driver package for what ever is going to be your primary network adapter under Windows 2000/XP, unless you are sure that it is natively supported by Windows 2000/XP

- Take contents of 		\3400setup\move2root\

and move the four files to the root of new drive, if you don’t see four files check your explorer’s tools/folder options and ensure that you aren’t hiding system and hidden files from yourself.

- Remove drive from desktop and install in Stylistic 3400/3500/LT-C500/LT-C600. Power up tablet, USB keyboard and mouse connected. Use bios to detect new drive if necessary

- At command prompt:		cd i386

winnt.exe

- Finish up basic install of Windows 2000/XP

********

Installation of drivers for your Stylistic 3400 / 3500 / LT-C500 / LT-C600

During this next phase, getting the drivers right, there will be lots of dialog boxes that I am not going to mention, for most of them you should take the default choice. Specifically there will be many warnings of “Driver not signed….Do you want to continue?” You must accept these and force windows to continue with the unsigned drivers or the tablet will not work properly

********

- Install drivers for devices in this order:

Video		Easy; RT click desktop, properties, settings, advanced, adapter ....

Audio		Easy; in device manager find impaired audio, update driver.

APM	tricky; first unknown device in device manager, update driver, direct search to BatteryAid directory

Now run setup.exe in the BatteryAid directory to install the utility

Digitizer	overall tricky!

Second and should be last/only unknown device in device manager, update, directing search to \digitizer\1_pen

***Dificult***	under the hardware tab of the mouse applet in control panel, select “Fujitsu stylistic 3400 pen”: FORCE driver update using moufiltr.ini in 2_CIC.

Note: FORCE by not letting driver wiz search, specify other, have disk, browse.

Run the setup in \digitizer\3_CIC_PenX2.02_APP  -  is utility that actually provides handwriting recognition, to be installed after the two previous steps are successful.

Browse to the \digitizer\4_SAS directory, Double click to excecute the RUNONCE.BAT batch program, you will be presented with a winzip dialog, click “unzip” in the upper right, click it then close the winzip. Next you will see a windows dialog confirming the registry write, click yes.

Next the SAS driver must be installed, follow these steps:

a. Double-click the Add/Remove Hardware applet in Control Panel.

b. Select Add/Troubleshoot a Device.

c. Select Add a new device.

d. Select No, I Want to Select the Hardware from a list.

e. Select Other Devices and then click Next.

f. Click Have Disk and browse to the directory \digitizer\4_SAS\  that contains the fujdrv.inf file.

****	A reboot here wouldn’t hurt  ****

After a reboot, there should be a small hand in the system tray, this little app invokes the WinNT/2K/XP ctrl-alt-del with a stylus tap, occasionally necessary, and quite difficult with no keyboard in slate mode.

- Install your PCMCIA wired or wireless network adapter.

- Change boot menu delay:

You probably notice at one or more of the reboots that you actually have a dual boot system, switchable between Windows 2000 Professional, and Windows XP, which is as you the know command line only Win98(dos7) that we used to setup the disk. While operating with the keyboard it’s not a problem, the enter key bypasses the 30 second delay and you are on your way. The first time you wait that 30 seconds because you have only a pen you will wish that you had done the following:

Right click My Computer, select Properties, choose the Advanced tab, click Startup and recovery: here you can either disable the list or drop the delay way down to 2 or 3 sec. you may want non-NT secured access to your drive one day, for recovery or virus removal. I leave mine on 4 sec, I am getting old and slow.

- Plain old windows maintenance, any available windows updates, customize your desktop, install software across your network.

That’s about it; it took me much longer to type this than to do it, Just about 2 hours, not bad considering the time spent formatting and disk diagnostics on the new drive while it was in the desktop.